Computer Networks CAT/ 20 Marks

- Provide the three main functions of data link layer

(3 marks)

Answer:

- Addressing

- Flow control

- Error control

- Media Access control.

- Define a protocol and explain three elements of a protocol.      (4 marks)

Answer:

- A protocol is identical with a rule. It consists of a set of rules that govern data communications. It determines what is communicated, how it is communicated and when it is communicated.

- Elements of a Protocol:

- Syntax: The term syntax refers to the structure or format of the data, meaning the order in which they are presented.

- Semantics: The word semantics refers to the meaning of each section of bits.

- Timing: The term timing refers to two characteristics: when data should be sent and how fast they can be sent.

- Differentiate distance vector routing protocols from link-state routing protocols.                                                                            (4 marks)

Answer:

| Distance vector routing protocols | Link-state routing protocols |
| --- | --- |
| Distance vector routing protocols are based on the Bellman-Ford algorithm. | Based on Dijkstra’s algorithm. |
| Distance vector routing protocols are concerned with the distance and vector, or direction, of destination networks. | Builds a complete database of all the link states of every router in its area. |
| Generally use less memory and processing power. | Benefits include faster convergence and improved bandwidth utilization over distance vector protocols |
| Relatively easy to configure. | Require complex administration. |

- Briefly describe two error correction methods.                         (4 marks)

Answer:

  - Vertical Redundancy Check (VRC): Perhaps the most familiar error-detecting code is the VRC (Parity Bit Method). In this code, a k-bit dataword is changed to an n-bit codeword where n= k+ 1. The extra bit, called the parity bit is selected to make the total number of 1s in the codeword even.

  - Longitudinal Redundancy Check (LRC): In Longitudinal redundancy check (LRC),a block of bits is organized in a table(rows and columns). Then calculate the parity bit for each column and create a new row of 8 bits, which are the parity bits for the whole block. First parity bit in the fifth row is calculated based on all first bits, and so on. Then attach the 8 parity bits to the original data and send them to the receiver.

  - Cyclic Redundancy Check (CRC):

Checksum:

- Provide one advantage and one dis-advantage of Dynamic Routing. (2 Marks)

- With help of diagrams, describe three types of guided transmission media. 3 marks